GEOID

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

The purpose of GEOID is to support conversions between WGS 84 ellipsoid heights, Earth Gravity Model 1996 (EGM 96) Geoid heights, and Earth Gravity Model 1984 (EGM 84) Geoid heights.

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of GEOID is to provide a reusable component that supports the following coordinate conversions:

- WGS 84 Ellipsoid height (meters) to EGM 96 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 15-minute geoid separation grid and a bilinear interpolation method,

- WGS 84 Ellipsoid height (meters) to EGM 96 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a variable resolution grid separation grid and a natural spline interpolation method,

- WGS 84 Ellipsoid height (meters) to EGM 84 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a bilinear interpolation method,

- WGS 84 Ellipsoid height (meters) to EGM 84 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a natural spline interpolation method,

- EGM 96 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 15-minute geoid separation grid and a bilinear interpolation method,

- EGM 96 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a variable resolution grid separation grid and a natural spline interpolation method,

- EGM 84 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a bilinear interpolation method, and

- EGM 84 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a natural spline interpolation method.

A particular variation of the Geoid conversion is specified in terms of the following parameters:

- Longitude – Longitude (in radians) at the height conversion point,

- Latitude – Latitude (in radians) at the height conversion point.

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Initialize_Geoid – This function reads the EGM96 and EGM84 Geoid separation data from files.

Convert_Ellipsoid_To_Geoid_Height – This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent EGM 96 geoid height using a 15-minute grid and bilinear interpolation.

Convert_Ellipsoid_To_MSL_EGM96_VG_NS_Height – This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent EGM 96 geoid height using a variable resolution grid and natural spline interpolation.

Convert_Ellipsoid_To_MSL_EGM84_10D_BL_Height – This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent EGM 84 geoid height using a 10-degree grid and bilinear interpolation.

Convert_Ellipsoid_To_MSL_EGM84_10D_NS_Height – This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent EGM 84 geoid height using a 10-degree grid and natural spline interpolation.

Convert_Geoid_To_Ellipsoid_Height – This function converts the specified EGM 96 geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height using a 15-minute grid and bilinear interpolation.

Convert_MSL_EGM96_VG_NS_To_Ellipsoid_Height – This function converts the specified EGM 96 geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height using a variable resolution grid and natural spline interpolation.

Convert_MSL_EGM84_10D_BL_To_Ellipsoid_Height – This function converts the specified EGM 84 geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height using a 10-degree grid and bilinear interpolation.

Convert_MSL_EGM84_10D_NS_To_Ellipsoid_Height – This function converts the specified EGM 84 geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height using a 10-degree grid and natural spline interpolation.

# EXAMPLE ApplicationS:

The following example illustrates how Geoid can be used to convert a WGS 84 ellipsoid height to the corresponding EGM 96 geoid height using bilinear interpolation and back again:

## Function Call:

status = Initialize_Geoid()

Inputs:

None

## Outputs:

- None

## Function Call:

status = Convert_Ellipsoid_To_Geoid_Height (Latitude, Longitude, Ellipsoid_Height, Geoid_Height)

## Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Ellipsoid Height:	0.0

## Outputs:

- Geoid Height:	43.0

## Function Call:

status = Convert_Geoid_To_Ellipsoid_Height (Latitude, Longitude, Geoid_Height, Ellipsoid_Height)

## Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Geoid Height:	43.0

## Outputs:

- Ellipsoid Height:	0.0